|  | MAINE LTC/SNF EMERGENCY POWER & CRITICAL UTILITIES WORKSHEET HCC-ME Long-Term Care / Skilled Nursing Continuity & Infrastructure Planning Tool |
| --- | --- |
| PLANNING RESOURCE This LTC/SNF worksheet is an optional planning and documentation aid. It does not replace current CMS emergency preparedness, CMS Life Safety Code, Maine DHHS/DLC, Maine Office of State Fire Marshal, local AHJ, electrical, generator, fuel-storage, environmental, or other applicable requirements. Current controlling requirements and qualified technical guidance govern. |  |  |

FACILITY INFORMATION

| Facility / Organization |  | Provider Type | LTC / SNF | Primary Site / Building |  |
| --- | --- | --- | --- | --- | --- |
| Preparedness / Facilities Lead |  | Date Completed / Reviewed |  | Next Review Date |  |
| Maine License # |  | CMS CCN |  | Licensed Bed Capacity |  |

HOW TO USE THIS WORKSHEET

Document utility sources, backup capabilities, critical resident-care dependencies, connection/isolation methods, vendors, and estimated duration of continuity.

Use this worksheet with the facility EOP, HVA, maintenance records, fire/life-safety documentation, resident dependency data, and qualified facilities/engineering guidance.

Pre-identify decision points for conservation, internal relocation, transfer, evacuation, service reduction, or closure before a prolonged utility failure occurs.

Keep completed operational copies in a controlled location. Do not enter resident-identifiable clinical information in a broadly distributed EOP copy.

LTC/SNF REGULATORY ALIGNMENT CMS 42 CFR §483.73(b)(1) requires policies/procedures for subsistence needs and alternate energy, including food, water, medical/pharmaceutical supplies, safe temperatures, emergency lighting, fire systems, and sewage/waste. Maine 10-144 CMR Ch. 110 includes additional emergency-power and facility utility requirements. Coordinate generator and life-safety compliance with applicable CMS LSC/K-tags, Maine FMO requirements, and adopted NFPA standards.

1. LTC/SNF CRITICAL UTILITIES - SOURCE, BACKUP & CAPACITY

| System / Utility | Primary Source | Backup / Alternate | Critical Areas / Functions Served | Estimated Runtime / Capacity |
| --- | --- | --- | --- | --- |
| Electrical Power |  |  |  |  |
| Generator / Emergency Power |  |  |  |  |
| Generator / Heating Fuel |  |  |  |  |
| Heating / HVAC / Required Heat |  |  |  |  |
| Potable Water |  |  |  |  |
| Water-Supply Pumps |  |  |  |  |
| Wastewater / Sewer / Septic Pumps |  |  |  |  |
| Oxygen / Medical Gas |  |  |  |  |
| Nurse Call / Resident Alert Systems |  |  |  |  |
| Telephone / Communications |  |  |  |  |
| Internet / Network / EHR |  |  |  |  |
| Fire Alarm / Sprinkler / Life Safety |  |  |  |  |
| Medication / Vaccine Refrigeration |  |  |  |  |
| Food Refrigeration / Kitchen |  |  |  |  |
| Elevators / Vertical Transport |  |  |  |  |
| Powered Beds / Lifts / Resident Equipment |  |  |  |  |
| Security / Access Control |  |  |  |  |
| Other Critical Utility |  |  |  |  |

2. CRITICAL UTILITIES - OPERATING & RESPONSE DETAILS

| System / Utility | Start / Transfer / Isolation Method | Vendor / 24-Hour Emergency Contact | Shutoff / Connection / Key Location | Testing / Inspection / Notes |
| --- | --- | --- | --- | --- |
| Electrical Power |  |  |  |  |
| Generator / Emergency Power |  |  |  |  |
| Generator / Heating Fuel |  |  |  |  |
| Heating / HVAC / Required Heat |  |  |  |  |
| Potable Water |  |  |  |  |
| Water-Supply Pumps |  |  |  |  |
| Wastewater / Sewer / Septic Pumps |  |  |  |  |
| Oxygen / Medical Gas |  |  |  |  |
| Nurse Call / Resident Alert Systems |  |  |  |  |
| Telephone / Communications |  |  |  |  |
| Internet / Network / EHR |  |  |  |  |
| Fire Alarm / Sprinkler / Life Safety |  |  |  |  |
| Medication / Vaccine Refrigeration |  |  |  |  |
| Food Refrigeration / Kitchen |  |  |  |  |
| Elevators / Vertical Transport |  |  |  |  |
| Powered Beds / Lifts / Resident Equipment |  |  |  |  |
| Security / Access Control |  |  |  |  |
| Other Critical Utility |  |  |  |  |

3. EMERGENCY POWER / GENERATOR DETAILS

| Generator Make / Model ____________________________ | Rated Capacity (kW/kVA) ____________________________ | Fuel Type ____________________________ | Fuel Capacity ____________________________ |
| --- | --- | --- | --- |
| Generator Location ____________________________ | Automatic / Manual Transfer ____________________________ | Transfer Switch Location ____________________________ | Emergency Connection / Quick Connect ____________________________ |
| Typical Load / Critical Circuits ____________________________ | Estimated Runtime at Typical Load ____________________________ | Refueling Threshold ____________________________ | Primary Fuel Vendor / 24-Hour Contact ____________________________ |
| Alternate Fuel Vendor / Source ____________________________ | Portable / Rental Generator Plan ____________________________ | Connection / Cable Requirements ____________________________ | Qualified Staff / Contractor ____________________________ |
| Last Preventive Maintenance ____________________________ | Last Load-Bank / Functional Test ____________________________ | Last Fuel Quality / Tank Check ____________________________ | Next Scheduled Service ____________________________ |

LTC/SNF-SPECIFIC EMERGENCY POWER CHECK

| Planning / Compliance Prompt | Status | Evidence / Notes |  |
| --- | --- | --- | --- |
| Life-support equipment/systems used in the facility? | ☐ Yes ☐ No | If yes, identify supported locations/equipment and continuity plan. |  |
| On-premises emergency generator supports required life-support systems, if applicable. | ☐ Verified ☐ N/A | Evidence / record location: ______________________________ |  |
| Emergency power supports essential resident-care and life-safety services identified by the facility. | ☐ Verified | Critical-load panel / circuit list location: __________________ |  |
| Maine monthly generator operation/testing requirement is tracked in maintenance records. | ☐ Verified | Last documented monthly run: __________ Record location: __________ |  |
| Applicable CMS LSC / NFPA inspection, testing, maintenance, and fuel records are current and retrievable. | ☐ Verified | Record location / responsible lead: __________________________ |  |
| IMPORTANT This worksheet is not the generator maintenance log and does not replace required CMS Life Safety Code/K-tag, NFPA, Maine FMO, or qualified technician inspection/testing records. Reference the controlled record location above. |  |  |

4. FUEL, WATER, OXYGEN & CRITICAL SUPPLY CONTINUITY

| Resource | Normal Supply / Vendor | Minimum Operating Level | Emergency Reorder / Activation Point | Alternate Source / Contingency |
| --- | --- | --- | --- | --- |
| Generator / Heating Fuel |  |  |  |  |
| Potable Water |  |  |  |  |
| Sanitation / Wastewater |  |  |  |  |
| Oxygen / Medical Gas |  |  |  |  |
| Medication / Vaccine Refrigeration |  |  |  |  |
| Food Refrigeration / Dietary Operations |  |  |  |  |
| Batteries / UPS / Portable Power |  |  |  |  |
| Other Critical Utility Supply |  |  |  |  |

SUPPLY / VENDOR VERIFICATION

| Verification Item | Owner / Contact | Last Verified / Notes |
| --- | --- | --- |
| Primary fuel delivery access during severe weather / road closure |  |  |
| Alternate water source and delivery method |  |  |
| Oxygen / medical gas emergency resupply and cylinder storage plan |  |  |
| Medication / vaccine cold-chain contingency |  |  |
| Food refrigeration / emergency dietary continuity |  |  |
| Wastewater / septic contingency and vendor access |  |  |

5. RESIDENT DEPENDENCY & CLINICAL CONTINUITY IMPACT

| USE AGGREGATE / OPERATIONAL DATA Use counts or planning categories rather than resident-identifiable information in the master EOP copy. Maintain resident-specific details in a secured Resident Special Needs / Evacuation Assistance Tracker or clinical record. |  |  |  |
| --- | --- | --- | --- |
| Resident / Care Dependency | Current Count / Areas | Backup / Manual Method if Utility Fails | Trigger for Relocation / Transfer |  |
| Oxygen-dependent residents |  |  |  |  |
| Ventilator / life-support system users |  |  |  |  |
| Electricity-dependent medical devices |  |  |  |  |
| Powered beds / lifts required for safe care |  |  |  |  |
| Dialysis-dependent residents |  |  |  |  |
| Medication / insulin refrigeration needs |  |  |  |  |
| Temperature-sensitive residents / high-acuity units |  |  |  |  |
| Residents requiring continuous nurse-call / monitoring |  |  |  |  |
| Bariatric / specialized movement equipment |  |  |  |  |
| Other utility-dependent resident need |  |  |  |  |

CLINICAL CONTINUITY OWNERS

| Function | Primary / Backup Lead | 24/7 Contact / Controlled Directory |
| --- | --- | --- |
| Nursing / DON |  |  |
| Facilities / Maintenance |  |  |
| Respiratory / Oxygen Vendor |  |  |
| Pharmacy |  |  |
| Dietary |  |  |
| Emergency Preparedness Lead |  |  |

6. CRITICAL LOAD / SERVICE PRIORITIZATION

| MAINE LTC/SNF PROMPT Maine Chapter 110 identifies essential services that must be considered during electrical outages, including fire detection/alarm, telephone, boiler components, exit/corridor lighting, call systems, nurses-station lighting, food preparation, required heat, water-supply pumps, and private septic pumps. Verify applicability to the facility and map each applicable service to its emergency-power or contingency strategy. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Priority | Critical Service / Area | Emergency Power / Utility Needed | Minimum Requirement / Duration | Contingency if Utility Fails |  |
| 1 | Fire detection / alarm / sprinkler support |  |  |  |  |
| 2 | Exit / corridor / emergency lighting |  |  |  |  |
| 3 | Nurse call / resident alert systems |  |  |  |  |
| 4 | Nurses-station / clinical work areas |  |  |  |  |
| 5 | Required heat / boiler burners-fans-pumps |  |  |  |  |
| 6 | Water-supply pumps |  |  |  |  |
| 7 | Private septic / wastewater pumps |  |  |  |  |
| 8 | Food preparation / dietary operations |  |  |  |  |
| 9 | Life-support / oxygen / medical equipment |  |  |  |  |
| 10 | Medication / vaccine refrigeration |  |  |  |  |
| 11 | Telephone / emergency communications |  |  |  |  |
| 12 | Other facility-specific critical load |  |  |  |  |

7. UTILITY FAILURE DECISION THRESHOLDS & ACTIONS

| Failure / Condition | Monitor / Conservation Threshold | Escalation / Relocation Threshold | Decision Authority | Immediate Actions / Notifications |
| --- | --- | --- | --- | --- |
| Loss of commercial power |  |  |  |  |
| Generator failure / reduced capacity |  |  |  |  |
| Fuel below emergency threshold |  |  |  |  |
| Loss of heat / unsafe temperature |  |  |  |  |
| Loss of potable water |  |  |  |  |
| Wastewater / septic failure |  |  |  |  |
| Oxygen / medical gas interruption |  |  |  |  |
| Nurse call / communications failure |  |  |  |  |
| Medication / food refrigeration failure |  |  |  |  |
| Prolonged elevator / mobility system outage |  |  |  |  |

DECISION FACTORS

Resident acuity and dependency on heat, oxygen, electricity, refrigeration, lifts, nurse call, or other critical systems.

Estimated outage duration versus generator runtime, fuel resupply, water, oxygen, staffing, transportation, and receiving-facility availability.

Ability to maintain safe temperatures, fire/life-safety systems, sanitation, medications, food/hydration, infection control, and essential resident care.

Local emergency conditions, utility/vendor restoration estimates, and direction from public safety, emergency management, Maine DLC, or other authorities as applicable.

8. OUTAGE RESPONSE / RESTORATION CHECK

|  | Response / Restoration Action |
| --- | --- |
| ☐ | Confirm outage scope and immediate resident / life-safety risks. |
| ☐ | Activate emergency power or utility contingency procedures as applicable. |
| ☐ | Confirm fire alarm, emergency lighting, nurse call, required heat, water/septic pumps, communications, and other applicable essential services are supported. |
| ☐ | Identify residents whose care depends on electricity, oxygen/medical gas, refrigeration, powered mobility equipment, or other affected systems. |
| ☐ | Initiate enhanced rounding / manual monitoring when nurse-call, alarm, EHR, communications, or other systems are degraded. |
| ☐ | Notify leadership, nursing, facilities, affected departments, utilities/vendors, and external partners as needed. |
| ☐ | Track generator fuel, water, oxygen/gas, batteries/UPS, refrigeration temperatures, and other time-limited resources. |
| ☐ | Implement conservation, alternate-care, internal relocation, transfer, evacuation, or service-reduction strategies when thresholds are reached. |
| ☐ | Document operational decisions, resident movement, vendor contacts, work orders, and significant system status changes. |
| ☐ | After restoration, test affected systems, reconcile downtime records, verify safe temperatures/clinical systems, and authorize return to normal operations. |

RESTORATION / VERIFICATION LOG

| Date / Time | System Restored / Tested | Restoration Source | Verified By | Issue Remaining? | Follow-Up / Work Order |
| --- | --- | --- | --- | --- | --- |

9. CONTINUITY DECISION POINTS / NOTES

ANNUAL / POST-CHANGE VERIFICATION

| Item | Verified By / Date | Changes / Follow-Up |  |
| --- | --- | --- | --- |
| Generator capability / critical loads |  |  |  |
| Fuel vendor / alternate fuel source |  |  |  |
| Water / wastewater contingency |  |  |  |
| Oxygen / medical gas continuity |  |  |  |
| Resident utility-dependency planning |  |  |  |
| Utility contacts / shutoff / connection locations |  |  |  |
| Decision thresholds / receiving & transportation arrangements |  |  |  |
| REVIEW NOTE Review this worksheet with the EOP/HVA at least on the facility’s established annual review cycle and after material changes to utilities, generator configuration, resident population, building systems, vendors, or emergency arrangements. Current regulatory and code requirements control. |  |  |